SmartLight Totem-Wiring Reference

# Stage 1 - Heltec WiFi LoRa 32 V3 on Breadboard

Place the Heltec board straddling the centre gap of the breadboard. The board occupies both sides of the board. Make sure the USB-C port faces outward for easy access.

Power connections

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Heltec 3.3V pin | Breadboard + rail (left) | Powers 3.3V components |
| Heltec GND pin | Breadboard – rail (left) | Common ground |
| Heltec 3.3V pin | Breadboard + rail (right) | Link both rails |
| Heltec GND pin | Breadboard – rail (right) | Link both rails |

⚠ Always link the left and right power rails together. Many wiring issues come from a component being on the wrong rail side.

Stage 2 - 12V RGB LED Strip + MOSFETs

Three IRLZ44N MOSFETs switch the red, green and blue channels of the LED strip. Each MOSFET has three legs: Gate (G), Drain (D), Source (S),  left to right when the flat face is towards you.

MOSFET - Red channel (GPIO19)

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Gate (G) | GPIO19 via 680Ω resistor | Signal from Heltec |
| Gate (G) | GND rail via 10kΩ resistor | Pull-down to prevent floating |
| Drain (D) | LED strip Red wire | Switches the red channel |
| Source (S) | GND rail | Common ground |

## MOSFET - Green channel (GPIO20)

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Gate (G) | GPIO20 via 680Ω resistor | Signal from Heltec |
| Gate (G) | GND rail via 10kΩ resistor | Pull-down to prevent floating |
| Drain (D) | LED strip green wire | Switches the green channel |
| Source (S) | GND rail | Common ground |

## MOSFET - Blue channel (GPIO21)

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Gate (G) | GPIO21 via 680Ω resistor | Signal from Heltec |
| Gate (G) | GND rail via 10kΩ resistor | Pull-down to prevent floating |
| Drain (D) | LED strip Blue wire | Switches the blue channel |
| Source (S) | GND rail | Common ground |

## 12V Power to LED strip

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| 12V adapter + | LED strip +12V wire | Direct connection |
| 12V adapter – | GND rail | Shared ground with Heltec |
| LED strip +12V | All three MOSFET Drain sides | Common +12V for all channels |

Stage 3 - TEMT6000 Ambient Light Sensor

The TEMT6000 is a small 3-leg sensor (looks like a transistor). Hold it with the flat face towards you, left leg is Emitter, middle is Base (signal), right is Collector.

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Left leg (Emitter) | GND rail | Ground |
| Middle leg (Base) | GPIO1 | Analog signal to Heltec |
| Right leg (Collector) | 3.3V rail | Power |

Stage 4 - SD Card Module

The SD card module uses a second SPI bus (separate from the LoRa radio) on GPIO4–7. This avoids conflicts with the LoRa SPI pins.

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| CS | GPIO7 | Chip select |
| SCK | GPIO5 | SPI clock |
| MOSI | GPIO6 | Data to SD card |
| MISO | GPIO4 | Data from SD card |
| VCC | 3.3V rail | Power |
| GND | GND rail | Ground |

⚠ Insert a FAT32-formatted microSD card before powering on. The module will not initialise without a card present.

GPIO Assignment Summary

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| GPIO1 | TEMT6000 signal | Analog input - light sensor |
| GPIO4 | SD card MISO | SPI2 data in |
| GPIO5 | SD card SCK | SPI2 clock |
| GPIO6 | SD card MOSI | SPI2 data out |
| GPIO7 | SD card CS | SPI2 chip select |
| GPIO19 | MOSFET Red gate | PWM / digital output |
| GPIO20 | MOSFET Green gate | PWM / digital output |
| GPIO21 | MOSFET Blue gate | PWM / digital output |

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| GPIO1 | TEMT6000 signal | Analog input - light sensor |
| GPIO4 | SD card MISO | SPI2 data in |
| GPIO5 | SD card SCK | SPI2 clock |
| GPIO6 | SD card MOSI | SPI2 data out |
| GPIO7 | SD card CS | SPI2 chip select |
| GPIO19 | MOSFET Red gate | PWM / digital output |
| GPIO20 | MOSFET Green gate | PWM / digital output |