SmartLight Totem- Wiring Reference

# Stage 1- Heltec WiFi LoRa 32 V3 on Breadboard

Place the Heltec board straddling the centre gap of the breadboard. The board occupies both sides of the board. Make sure the USB-C port faces outward for easy access.

## Power connections

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Heltec 3.3V pin | Breadboard + rail (left) | Powers 3.3V components |
| Heltec GND pin | Breadboard – rail (left) | Common ground |
| Heltec 3.3V pin | Breadboard + rail (right) | Link both rails |
| Heltec GND pin | Breadboard – rail (right) | Link both rails |

⚠ Always link the left and right power rails together. Many wiring issues come from a component being on the wrong rail side.

Stage 2 -12V RGB LED Strip + MOSFETs

Three IRLZ44N MOSFETs switch the red, green and blue channels of the LED strip. Each MOSFET has three legs: Gate (G), Drain (D), Source (S)-left to right when the flat face is towards you.

## MOSFET- Red channel (GPIO19)

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Gate (G) | GPIO19 via 680Ω resistor | Signal from Heltec |
| Gate (G) | GND rail via 10kΩ resistor | Pull-down to prevent floating |
| Drain (D) | LED strip Red wire | Switches the red channel |
| Source (S) | GND rail | Common ground |

## MOSFET Green channel (GPIO20)

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Gate (G) | GPIO20 via 680Ω resistor | Signal from Heltec |
| Gate (G) | GND rail via 10kΩ resistor | Pull-down to prevent floating |
| Drain (D) | LED strip Green wire | Switches the green channel |
| Source (S) | GND rail | Common ground |

## MOSFET - Blue channel (GPIO21)

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Gate (G) | GPIO21 via 680Ω resistor | Signal from Heltec |
| Gate (G) | GND rail via 10kΩ resistor | Pull-down to prevent floating |
| Drain (D) | LED strip Blue wire | Switches the blue channel |
| Source (S) | GND rail | Common ground |

## 12V Power to LED strip

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| 12V adapter + | LED strip +12V wire | Direct connection |
| 12V adapter – | GND rail | Shared ground with Heltec |
| LED strip +12V | All three MOSFET Drain sides | Common +12V for all channels |

Stage 3 -TEMT6000 Ambient Light Sensor

The TEMT6000 is a small 3-leg sensor (looks like a transistor). Hold it with the flat face towards you — left leg is Emitter, middle is Base (signal), right is Collector.

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Left leg (Emitter) | GND rail | Ground |
| Middle leg (Base) | GPIO1 | Analog signal to Heltec |
| Right leg (Collector) | 3.3V rail | Power |

⚠ GPIO1 has some noise from USB serial interference. The hysteresis thresholds in the code (TURN_ON: 50, TURN_OFF: 400) compensate for this. Do not move to GPIO2 or GPIO45 — these showed worse results in testing.

# Stage 4 - SD Card Module

The SD card module uses the second SPI bus on GPIO33–36, separate from the LoRa radio pins. VCC must be connected to the 5V (VEXT) pin — 3.3V is not sufficient for this module.

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| CS | GPIO33 | Chip select |
| SCK | GPIO34 | SPI clock |
| MOSI | GPIO35 | Data to SD card |
| MISO | GPIO36 | Data from SD card |
| VCC | VEXT (5V pin) | Power — must be 5V not 3.3V |
| GND | GND rail | Ground |

⚠ Insert a FAT32-formatted microSD card before powering on. The module will not initialise without a card present. Original GPIO4–7 assignment was found incompatible — use GPIO33–36 only.

# Stage 5 - NeoPixel Ring 12 (Status Indicator)

The NeoPixel Ring 12 acts as the flood alert and system status indicator. It has three pads: 5V DC Power, GND, and Data Input. The Data Input pad receives the signal from the Heltec. Do not connect to Data Out — that is only used when chaining multiple rings together.

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| 5V DC Power | VEXT (5V pin) | Power — must be 5V |
| GND | GND rail | Ground |
| Data Input (DIN) | GPIO38 | Signal from Heltec |

⚠ Always connect to Data Input (DIN), not Data Out (DOUT). The ring requires 5V power — 3.3V is insufficient.

Stage 6 -INA219 Power Monitor

The INA219 measures voltage, current and power on the 12V supply line. It connects via I2C and sits in series on the positive 12V wire — current flows through it so it can measure the load. The module appears at I2C address 0x41 (not the default 0x40).

## I2C connections

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| VCC | 3.3V rail | Power |
| GND | GND rail | Ground |
| SDA | GPIO41 | I2C data |
| SCL | GPIO42 | I2C clock |

## In-line current measurement

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| VIN+ | 12V adapter positive wire | Current flows in here |
| VIN– | LED strip +12V wire | Current flows out to load |

⚠ VIN+ and VIN-must be wired in series with the 12V supply -not both connected to the same rail. The I2C address is 0x41, not the default 0x40. Always specify ina219(0x41) in code.

# Stage 7-Push Button (State Machine / USB Charging Trigger)

A momentary push button on GPIO47 triggers the USB charging session (30-minute timed window). It uses the Heltec's internal pull-up resistor - no external resistor is needed. One leg connects to GPIO47, the other to GND.

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| One leg | GPIO47 | Signal to Heltec |
| Other leg | GND rail | Ground |

⚠ Use a momentary (push-to-make) button, not a toggle switch. The code uses INPUT_PULLUP so no external resistor is needed. For the final totem, use a vandal-resistant 16mm metal momentary push button.

# Stage 8-USB Charging Port (Pending MOSFET Delivery)

The USB charging port (INPUT: DC 12-24V, OUTPUT: DC 5V 3.1A) is switched by a fourth IRLZ44N MOSFET on GPIO48. The port's built-in regulator converts 12V to 5V internally. The MOSFET controls whether 12V reaches the port. A 30-minute timer in the state machine cuts power automatically after each session.

## MOSFET -USB charging (GPIO48)

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Gate (G) | GPIO48 via 680Ω resistor | Signal from Heltec |
| Gate (G) | GND rail via 10kΩ resistor | Pull-down to prevent floating |
| Drain (D) | USB port red wire (+) | Switches 12V to port |
| Source (S) | 12V rail | 12V supply |

## USB port connections

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| Red wire (+) | MOSFET Drain | 12V switched input |
| Black wire (–) | GND rail | Ground |

⚠ IRLZ44N MOSFET on order. Temporary use of IRF520 is possible but it only partially conducts at 3.3V gate voltage - use only for testing, not the final build.

# Stage 9 -DS3231 RTC Module (On Order)

The DS3231 Real Time Clock module provides accurate timestamps for SD card data logging. It connects via I2C on the same bus as the INA219 (shared GPIO41/42). The module has its own coin battery (CR2032) to maintain time when the main power is off. Default I2C address is 0x68.

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| VCC | 3.3V rail | Power |
| GND | GND rail | Ground |
| SDA | GPIO41 | I2C data (shared with INA219) |
| SCL | GPIO42 | I2C clock (shared with INA219) |

⚠ DS3231 is on order. I2C address 0x68 — no conflict with INA219 at 0x41. Ensure a CR2032 coin battery is installed in the module to preserve time across power cycles.

# GPIO Assignment Summary

| Component Pin | Connects To | Notes |
| --- | --- | --- |
| GPIO1 | TEMT6000 signal | Analog input — light sensor |
| GPIO19 | MOSFET Red gate | PWM / digital output — LED strip |
| GPIO20 | MOSFET Green gate | PWM / digital output — LED strip |
| GPIO21 | MOSFET Blue gate | PWM / digital output — LED strip |
| GPIO33 | SD card CS | SPI2 chip select |
| GPIO34 | SD card SCK | SPI2 clock |
| GPIO35 | SD card MOSI | SPI2 data out |
| GPIO36 | SD card MISO | SPI2 data in |
| GPIO38 | NeoPixel ring DIN | Status indicator data |
| GPIO41 | I2C SDA | INA219 + DS3231 data |
| GPIO42 | I2C SCL | INA219 + DS3231 clock |
| GPIO47 | Push button | USB charging trigger |
| GPIO48 | MOSFET USB gate | USB charging MOSFET (pending) |
| VEXT (5V) | SD card VCC + NeoPixel 5V | 5V output pin on Heltec |

⚠ Reserved/avoid: GPIO0 (boot), GPIO3 (USB serial), GPIO2 and GPIO45 (unstable analog). LoRa radio and OLED use fixed internal GPIOs -do not reassign. GPIO4–7 found incompatible with SD card on this board revision.